Enterprise Network Design — Multi-Area OSPF Lab

Author: Hasan Elfanhry  |  Tool: Cisco Packet Tracer  |  Level: CCNA

# 1. Overview

This lab simulates a full enterprise network built in Cisco Packet Tracer. It covers multi-area OSPF routing, VLANs, inter-VLAN routing via multilayer switches, centralized network services (DHCP, DNS, TFTP), and internet connectivity through static routing.

# 2. Network Topology

The network is divided into 4 OSPF areas:

Area 0 — Backbone: Central hub connecting all areas, contains the ASBR for internet access.

Area 1 — Building 1: First corporate building with 4 VLANs across 2 floors.

Area 2 — Building 2: Second corporate building with 4 VLANs across 2 floors.

Area 3 — Data Center: Centralized servers providing DHCP, DNS, and TFTP services.

# 3. Area 0 — Backbone

Area 0 is the OSPF backbone area. All other areas connect to it through ABRs (Area Border Routers). The ASBR router in this area has a static default route pointing to the internet:

ip route 0.0.0.0 0.0.0.0 <next-hop-IP>

The default route is then distributed to all OSPF areas using:

default-information originate

# 4. Area 1 — Building 1

Contains the first corporate building with two floors, each served by an access switch.

## VLAN Configuration

| VLAN ID | Name | Subnet | SVI Gateway | Department |
| --- | --- | --- | --- | --- |
| 10 | HR | 192.168.10.0/24 | 192.168.10.1 | Human Resources |
| 20 | Engineering | 192.168.20.0/24 | 192.168.20.1 | Engineering |
| 30 | Sales | 192.168.30.0/24 | 192.168.30.1 | Sales |
| 100 | Printers | 192.168.100.0/24 | 192.168.100.1 | Network Printers |
| 200 | Native | — | — | Trunk Native VLAN |

## Key Configurations

Inter-VLAN routing via multilayer switch SVIs (no external router needed).

VTP domain: "area1" — multilayer switch acts as VTP Server; access switches are VTP Clients.

Trunking between multilayer switch and access switches with Native VLAN 200.

PCs receive dynamic IPs from the centralized DHCP server (10.10.20.10) via ip helper-address on each SVI.

Printers are assigned static IPs for predictable addressing.

DNS A Records configured for all printers for easy name-based access.

# 5. Area 2 — Building 2

Contains the second corporate building. Same architecture as Building 1 with a different IP scheme.

## VLAN Configuration

| VLAN ID | Name | Subnet | SVI Gateway | Department |
| --- | --- | --- | --- | --- |
| 40 | IT | 172.31.40.0/24 | 172.31.40.1 | IT Department |
| 50 | Security | 172.31.50.0/24 | 172.31.50.1 | Security |
| 60 | Finance | 172.31.60.0/24 | 172.31.60.1 | Finance |
| 100 | Printers | 172.31.100.0/24 | 172.31.100.1 | Network Printers |
| 200 | Native | — | — | Trunk Native VLAN |

# 6. Area 3 — Data Center

The data center is isolated in Area 3 and hosts all centralized network services. It uses VLAN 90 with an SVI gateway of 10.10.20.1.

## Servers

| Service | IP Address | Role |
| --- | --- | --- |
| DHCP Server | 10.10.20.10 | Assigns dynamic IPs to all PCs network-wide |
| DNS Server | 10.10.20.20 | Resolves printer hostnames to IP addresses |
| TFTP Server | 10.10.20.30 | File transfer and IOS backup |

# 7. Technologies Used

OSPF Multi-Area Routing (Area 0, 1, 2, 3)

VLANs with 802.1Q Trunking

Inter-VLAN Routing via Layer 3 Switch SVIs

VTP (VLAN Trunking Protocol) — Server/Client mode

DHCP with ip helper-address relay agent

DNS — A Records for static-IP devices (printers)

Static default routing + default-information originate for internet

Native VLAN 200 for trunk security

# 8. Verification & Testing

| Test | Command / Method | Result |
| --- | --- | --- |
| OSPF routes on all routers | show ip route | All OSPF routes present ✅ |
| DHCP lease on PCs | ipconfig | Dynamic IP assigned ✅ |
| Cross-area connectivity | ping (Area 1 → Area 2) | Successful ✅ |
| DNS name resolution | ping printer-f1-b1 | Resolved and replied ✅ |
| Internet access | ping internet server | Successful ✅ |

Built as part of CCNA certification preparation — Hasan Elfanhry